16CXX PRINTER PORT PIC PROGRAMMER

By Ken Segler

of

CyberTech Computer Works

4308 YORKWOOD DR.

NORTH LAS VEGAS, NV 89030

USE THIS SOFTWARE AT YOUR OWN RISK CCW IS NOT LIABLE FOR ANY DAMAGE

WHATSOEVER FROM USING THIS SOFTWARE.

After building EDTech's 16c5x programmer and getting started in writing

a few PIC programs, I started looking at the other PIC's, ie. 16CXX series and

of course also looking at the available development tools and soon found out

there was no programmers that fit the bill (low cost).

So first I downloaded Russ Reiss's software for his programmer because I

liked his hardware setup, but after running the software once I gave up on that

idea and started looking again.

Then after logging on the MicroChip BBS and downloading David Tait's File

PIC84PGM.ZIP and immediately started looking through my stack of old pcb's

for a 7407, and had it built and going in no time just had to wait for UPS to

deliver a 16C84.(Thanks to David for a great programmer)

But being the kind to never leave things alone. I soon started to come up

with more ideas to make it a little more useful, so i found out that the 16CXX

series use basically the same serial programming setup, except for the timing

changes from EEPROM (10ms) and EPROM (100�s) programming times.

So I Started by rewriting the software in assembly code to get the speed

and accurate timing delays, and soon added support for the 16C64 and 16C71

and the option of adding other 16CXX support by leaving space in the program

for end user add in's.

Well here's the info on this software :

Can use Tait or Reiss's hardware or your own.

Reads, Verify's and Programs 16CXX PIC's

Programs 16C84's in under 10 seconds with 10ms delays

Programs : 16c61,c62,c63,c64,c65,c71,c73,c74,c84,c620,c621,c622

Registration Fee : Software $25.00

PCB      $20.00 late Aug

Registered Version Info :

Save Hex File, Stores Timer Values In .INI file

No Hit Enter To Continue

PGM16CXX.INI File Info :

The INI file is just a plain ascii text file, with no spaces

before or after the equals sign.

When you enter a filename on the command line PGM16CXX will

look for a INI file with the same name but with a INI extension

this file can include any or all of the following commands

PGM16CXX.INI is always loaded first.

Example PGM16CXX.INI File:

All Values in HEX

;�����������������������������������������������������������������������������

; PGM16CXX.INI

;

; Last UpDate  Tue  08/15/95 / 11:13 am

;�����������������������������������������������������������������������������

INFO='DEFAULT PGM16CXX.INI' ; This text will be displayed at line 25 of the

; display enter any text relevant to you

PRGEXIT=FALSE               ; Set to true if you want to program and exit

; to dos automatically

DEVICE=16C622               ; Sets device type

PORT=378                    ; Port address in hex 278,378,3bc

MSDELAY=14                  ; Eeprom program delay in ms

OVERPROG=3                  ; Eprom number of over programming pulses

ID1=7F                      ; ID1 in Hex

ID2=7F                      ; ID2 in Hex

ID3=7F                      ; ID3 in Hex

ID4=7F                      ; ID4 in Hex

OSC=RC                      ; Oscillator type LP, XT, HS, RC

WATCHDOG=OFF                ; On:Off

POWERUPTIMER=OFF            ; On:Off

CODEPROTECT=OFF             ; On:off

DATAIN=40                   ; Datain mask

DATAINV=0                   ; Data Invert Mask

VPPOFF=8                    ; Vpp Off

VPPON=0                     ; Vpp On

VDDOFF=4                    ; Vdd Off

VDDON=0                     ; Vdd On

CLKLO=0                     ; Clock Lo

CLKHI=2                     ; Clock Hi

OUTLO=0                     ; Out Lo

OUTHI=1                     ; Out Hi

Example CLOCK.INI File

;�����������������������������������������������������������������������������

; CLOCK.INI

;

; Last UpDate  Tue  08/15/95 / 11:13 am

;�����������������������������������������������������������������������������

INFO='Simple Lcd Clock For 16C84 Pic'

PRGEXIT=TRUE

DEVICE=16C84

ID1=01

ID2=02

ID3=03

ID4=04

OSC=RC

WATCHDOG=OFF

POWERUPTIMER=ON

CODEPROTECT=ON

Hardware Settings:

7407 & PNP  7407 & 4066  7406 & PNP  7406 & 4066

DATAIN       40          40           40          40

DATAINV      0           0            40          40

VPPOFF       8           0            0           8

VPPON        0           8            8           0

VDDOFF       4           0            0           4

VDDON        0           4            4           0

CLKLO        0           0            2           2

CLKHI        2           2            0           0

OUTLO        0           0            1           1

OUTHI        1           1            0           0

Russ Reiss Hardware Info:

To Use Reiss's Hardware just rewire the cable or make DB-25F to DB-25M

like this one. i haven't tested this but the design should work.

Use the 7407 and PNP Transistor Setup.

PcPort  PicSPA84

DATAI   2       3

CLOCK   3       2

VDD     4       14

VPP     5       1

DATAO   10      11

GND     25      25