PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD

*** PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD ***

Version  : 00

Revision : 00

Hardware : P942700-0 by FARSOFT COMPUTERS CC

Software : PP.EXE by David Tait

Author   : V00R00 - Robert Farrer

Date     : V00R00 - 4 July 1994

FARSOFT COMPUTERS CC, CK90/09203/23, ROBERT FARRER, 828-9255   PAGE 1

PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD

TABLE OF CONTENTS

=================

TITLE PAGE  . . . . . . . . . . . . . . . . . . . . . . 1

TABLE OF CONTENTS . . . . . . . . . . . . . . . . . . . 2

01. INTRODUCTION  . . . . . . . . . . . . . . . . . . . . . 3

02. PURPOSE OF MANUAL . . . . . . . . . . . . . . . . . . . 3

03. DISCLAIMERS . . . . . . . . . . . . . . . . . . . . . . 3

HARDWARE . . . . . . . . . . . . . . . . . . . . . . 3

SOFTWARE . . . . . . . . . . . . . . . . . . . . . . 3

MANUAL . . . . . . . . . . . . . . . . . . . . . . . 4

04. WARRANTY  . . . . . . . . . . . . . . . . . . . . . . . 4

05. COPYRIGHT . . . . . . . . . . . . . . . . . . . . . . . 4

06. HARDWARE  . . . . . . . . . . . . . . . . . . . . . . . 4

CABLE . . . . . . . . . . . . . . . . . . . . . . . . . 4

COMPONENT LIST  . . . . . . . . . . . . . . . . . . . . 5

RESISTORS  . . . . . . . . . . . . . . . . . . . . . 5

CAPACITORS . . . . . . . . . . . . . . . . . . . . . 5

SEMICONDUCTORS . . . . . . . . . . . . . . . . . . . 5

MISCELLANEOUS  . . . . . . . . . . . . . . . . . . . 5

ADJUSTMENT  . . . . . . . . . . . . . . . . . . . . . . 6

07. SOFTWARE  . . . . . . . . . . . . . . . . . . . . . . . 6

08. COMMENTS OR SUGGESTIONS OR FAULTS . . . . . . . . . . . 6

09. THE CAULDRON BBS  . . . . . . . . . . . . . . . . . . . 6

10. PIC16C5X MICROCONTROLLER COURSE AND OTHER PRODUCTS  . . 7

11. PRICE LIST  . . . . . . . . . . . . . . . . . . . . . . 7

FARSOFT COMPUTERS CC, CK90/09203/23, ROBERT FARRER, 828-9255   PAGE 2

PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD

01. INTRODUCTION

================

David Tait, of MANCHESTER UNIVERSITY, designed a low cost

PIC16C84 microcontroller programmer. David has, generously,

made his hardware and software design available, free of

charge.

The PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD provides a

convenient way of building the hardware described by David.

02. PURPOSE OF MANUAL

=====================

The purpose of this manual is to discuss the assembly of the

PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD.

The manual is not intended to teach the USER about PIC16C84

microcontrollers, electronics or any other technique which

may be mentioned in the course of this manual.

A working knowledge of electronics and IBM, (or compatible),

computers is required in order to use this manual.

03. DISCLAIMERS

===============

HARDWARE

--------

The basic PIC16C84 PROGRAMMER hardware was originally

designed by David Tait and is subject to the terms and

conditions described in his documentation.

FARSOFT COMPUTERS added the power supply section to the

basic design and then designed a printed circuit board,

(part number P942700-0), to implement the modified design.

FARSOFT COMPUTERS believes that the power supply and printed

circuit board are technically correct. However, FARSOFT

COMPUTERS reserves the right to revise the power supply and

printed circuit board and make periodic changes to the design

without a direct or inferred obligation to FARSOFT COMPUTERS

to notify any person of such revisions or changes.

FARSOFT COMPUTERS does not assume responsibility for

the use of the PIC16C84 PROGRAMMER hardware or the

PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD.

SOFTWARE

--------

The PIC16C84 PROGRAMMER software was originally designed

by David Tait and is subject to the terms and conditions

described in his documentation.

FARSOFT COMPUTERS does not assume responsibility for the use

of the PIC16C84 PROGRAMMER software.

FARSOFT COMPUTERS CC, CK90/09203/23, ROBERT FARRER, 828-9255   PAGE 3

PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD

MANUAL

------

This manual is compiled by FARSOFT COMPUTERS CC.

FARSOFT COMPUTERS believes that the information

contained in the manual is correct. However, FARSOFT

COMPUTERS reserves the right to revise the manual

and make periodic changes to the content without a

direct or inferred obligation to FARSOFT COMPUTERS

to notify any person of such revisions or changes.

FARSOFT COMPUTERS does not assume responsibility

for use of this manual.

04. WARRANTY

============

The PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD, (part number

P942700-0), is warranted against defects in material and

workmanship for a period of 30, (thirty), days from the date

of purchase by the original purchaser.

Your sales docket or invoice is your only proof of warranty

entitlement and must be produced when making a warranty

claim.

This warranty does not apply if the PIC16C84 PROGRAMMER

PRINTED CIRCUIT BOARD has been subject to static discharge,

misuse, neglect or accident.

05. COPYRIGHT

=============

The PIC16C84 PRINTED CIRCUIT BOARD design, (part number

P942700-0), is copyright by FARSOFT COMPUTERS CC.

However, in keeping with the spirit of David's package,

FARSOFT COMPUTERS CC grants permission to use, copy or

redistribute this design, so long as it is not sold for

profit.

06. HARDWARE

============

CABLE

-----

The Following cable is required to connect the PIC16C84

PROGRAMMER PRINTED CIRCUIT BOARD to the parallel port

of an IBM PC computer.

SEE DIAGRAM ON NEXT PAGE

FARSOFT COMPUTERS CC, CK90/09203/23, ROBERT FARRER, 828-9255   PAGE 4

PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD

<-- TO PROGRAMMER                               TO IBM PC -->

PIN   SIGNAL                                     SIGNAL   PIN

1 - Dout >-------------------------------------> !Ack - 10

2 - Clock <--------------------------------------< D1 -  3

3 - Program <------------------------------------< D3 -  5

4 - Din <----------------------------------------< D0 -  2

5 - Supply <-------------------------------------< D2 -  4

6 - Ground ----------------------------------- Ground - 22

7 - Ground ----------------------------------- Ground - 23

8 - Ground ----------------------------------- Ground - 24

9 - Ground ----------------------------------- Ground - 25

CN2 <---------------- 2m maximum -----------------> CN3

NOTE: All four ground connections are not required.

COMPONENT LIST

--------------

RESISTORS

R1 - 10K, 0.25W, 5%

R2 - 10K, 0.25W, 5%

R3 - 10K, 0.25W, 5%

R4 - 10K, 0.25W, 5%

R5 - 10K, 0.25W, 5%

R6 - 10K, 0.25W, 5%

R7 - 240E, 0.25W, 1%

R8 - 5K trimpot, multi-turn, side adjust

CAPACITORS

C1 - 10n, ceramic

C2 - 10n, ceramic

C3 - 100n, ceramic

C4 - 100n, ceramic

C5 - 220u, electrolytic, 25V, radial

C6 - 220u, electrolytic, 25V, radial

C7 - 47u, electrolytic, 16V, radial

SEMICONDUCTORS

IC1 - 7407 or 7406, (see section 07. SOFTWARE)

IC2 - 4066

IC3 - PIC16C84, (this is the IC being programmed)

B1 - W04, bridge rectifier

V1 - 78L05, voltage regulator

V2 - LM317, voltage regulator

MISCELLANEOUS

ST1 - 2 way, screw terminal, PCB mount

CN1 - 9 way, D-connector, female, right angle, PCB mount

CN2 - 9 way, D-connector, male, solder pot, with cover

CN3 - 25 way, D-connector, male, solder pot, with cover

9 core cable, multi-strand

IC socket for IC1, 14 pin

IC socket for IC2, 14 pin

IC socket for IC3, 18 pin, Zero Insertion Force, (ZIF)

FARSOFT COMPUTERS CC, CK90/09203/23, ROBERT FARRER, 828-9255   PAGE 5

PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD

ADJUSTMENT

----------

Solder all the components into the printed circuit board,

but do not insert the ICs into the sockets, until R8 has

been adjusted.

Connect 15Vac or 20Vdc to ST1 and adjust R8 until 13.8V

is measured between pins 7 and 14 of IC2.

07. SOFTWARE

============

If the PIC16C84 PROGRAMMER is built using a 7406 for IC1,

then the program PP.EXE may be used.

If the PIC16C84 PROGRAMMER is built using a 7407 for IC1,

then the source code PP.C must be edited and compiled.

QBASIC versions of the program are also included.

08. COMMENTS OR SUGGESTIONS OR FAULTS

=====================================

FARSOFT COMPUTERS is continually striving to improve its

products, if you have any comments or suggestions or have

detected any faults in the printed circuit board or manual,

please describe them in as much detail as possible and post

them to:

ATTENTION: ROBERT FARRER

FARSOFT COMPUTERS CC

P.O. Box 6334

Homestead

1412

South Africa

09. THE CAULDRON BBS

====================

FARSOFT COMPUTERS operates a bulletin board system,

(BBS), for product support.

Currently there is no charge for accessing the BBS,

however this may change in the future.

As you have expressed an interest in the PIC16C84

PROGRAMMER PRINTED CIRCUIT BOARD, the PIC USERS GROUP

may be of particular interest to you.

To contact THE CAULDRON BBS set your modem to 300, 1200

or 2400 bps, 8 data bits, no parity bit and 1 stop bit,

then dial 011-828-6731, there are two hunting lines

waiting for your calls.

To access the PIC USERS GROUP, type SSPIC and press return,

from the main menu prompt, then simply select options from

the menu.

FARSOFT COMPUTERS CC, CK90/09203/23, ROBERT FARRER, 828-9255   PAGE 6

PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD

10. PIC16C5X MICROCONTROLLER COURSE AND OTHER PRODUCTS

======================================================

FARSOFT COMPUTERS offers correspondence and part-time courses

on the PIC16C5X, MCS51 and Z80 microprocessors.

FARSOFT COMPUTERS is involved in the design and manufacture

of custom electronic, microprocessor and software systems.

If you require more information please contact

FARSOFT COMPUTERS:

By telephone:

ROBERT FARRER

FARSOFT COMPUTERS CC

011-828-9255

By post:

ATTENTION: ROBERT FARRER

FARSOFT COMPUTERS CC

P.O. Box 6334

Homestead

1412

South Africa

11. PRICE LIST

==============

1. All sales subject to FARSOFT COMPUTERS CC conditions

of sale, which are available on request.

2. All prices subject to change, without notice.

3. Remember FARSOFT COMPUTERS CC accepts VISA and

MASTER CARD, so you may pay your purchase off

over 6, 12, 18 or 24 months on your budget account.

4. Each postal order is subject to postage and

packaging of R15.00, (R13.16 + 14% VAT).

5. Call Robert or Susanne Farrer on 011-828-9255 to

place your order or request more information.

The PIC16C84 PROGRAMMER PRINTED CIRCUIT BOARD is only

available un-populated:

1 piece  - R28.50, (R25.00 + 14% VAT), each

10 pieces - R19.38, (R17.00 + 14% VAT), each

50 pieces - R14.82, (R13.00 + 14% VAT), each

Please note, in order to reduce costs, only one set of

documentation is supplied with each order, (on flexible

disc).

FARSOFT COMPUTERS CC, CK90/09203/23, ROBERT FARRER, 828-9255   PAGE 7