PP84.DOC

PP84 programmer software (release 25/6/96) usage notes

Steve Marchant, CCC, Nottingham university

This software id FreeWare, use it and copy it at will.

PP84 with no parameters will reset the in-system PIC

PP84 with the name of an 8-bit HEX file will read in the file,

validate it, report its 16 bit code check-sum, then proceed

to program the embedded data into the attached PIC16C84 device.

Data words at locations below $400 are programmed into code memory.

Data nibbles at locations $2000-2004 are programmed into ID memory.

Any Data item at location $2007 is programmed into the Config word.

Note that code protection is delayed 'til the end of programming.

Data bytes at locations $2100-$213F are programmed into EE data memory.

Programming errors are displayed to the screen, but programming continues.

A code protected device will automatically be bulk erased.

Only the locations specified the HEX file are programmed & verified.

A location which already contains the correct data will not be programmed.

The software will not start unless a PROG84 adapter is connected to LPT1.

History:

13/06/96 - Initial release

25/06/96 - Added a verify pass + minor cosmetic modifications

26/06/96 - Fixed error reporting bugs + PROG. now aborts on 5th error

To Do List:

* Look for adapter on LPT2 also.

* Option to save PIC memory image to HEX file.

* Option to override OSC settings in config word.

* Help Screen.

* Option to Verify Only.

Bugs:

Please report to: Steve.Marchant@Nottingham.ac.uk

###################################################################